SPECIFICATIONS FOR

REQUEST FOR QUOTATION

AND DELIVERY SCHEDULES

FOR PURCHASE OF

FIBER TERMINATION PANELS

and

FIBER PATCH PANELS

for

THE UNIVERSITY OF CALIFORNIA AT SANTA BARBARA

September, 1995

Department of Communications Services

University of California

at

Santa Barbara

# SECTION 1     PURCHASING LETTER

## 1.1     Purpose

The purpose of this Request For Quotation (RFQ) is to provide the University of California, Santa Barbara campus (UCSB), with the information to be used in the purchase of fiber termination panels and fiber patch panels.

## 1.2     Issuing Office

This RFQ is issued  for the  University by  the  Purchasing Department. Quotations must  be received by 4:30 p.m. on Monday, September 25, 1995  and submitted in three (3) copies to:

Oscar Woolfolk

Purchasing Department

University of California

Santa Barbara, CA 93106

(805) 893-3086

Technical Contact:  David Chapman  (805) 893-4030

### 1.3    Terms and Conditions

The terms and conditions specified in Appendix A shall apply.

### 1.4     Definitions

For the sake of brevity, the following definitions should be used whenever  the following  terms are used throughout the RFQ:

Multi-Mode Bulkhead Connectors:

The ST-to-ST  bulkhead connectors (also called "barrel connectors") provided within the multi-mode fiber termination and fiber patch panels for multi-mode fiber attachment.

Single Mode Bulkhead Connector (Optional Additive Bid):

The 568SC bulkhead connector (also called "barrel connectors") provided within the single mode fiber termination and fiber patch panels for a single mode single fiber attachment.

568SC Adapter  (Optional Additive Bid):

The 568SC adapters defined by EIA/TIA 568 as a mechanical termination device designed to align and join two like optical connectors.

Contract:

The  legal document  executed  between  the University and the Vendor covering the furnishing of fiber termination panels and fiber patch panels to the University.

Fiber Optic Segment:

A continuous section of fiber optic cable, each end of which terminates in ST-type (and optionally SC-type) connectors attached to ST-ST (and optionally SC) bulkhead adapters in each fiber termination and patch panel.

Multi-Mode Fiber Termination Panel (FTP):

The wall-mounted, forty-eight (48) fiber capacity dual access panel containing ST-typebulkhead connectors and splice trays to be acquired through this RFQ.

Multi-Mode Fiber Building Patch Panel (BPP):

The wall-mounted, twenty-four (24) fiber capacity dual access panels containing ST-type bulkhead connectors and splice trays to be acquired through this RFQ.

Multi-Mode Fiber Departmental Patch Panel (DPP):

The wall-mounted, twelve (12) fiber capacity dual access panels containing ST-type bulkhead connectors and splice trays to be acquired through this RFQ.

Single Mode Fiber Patch Panel (STP) (Optional Additive Bid):

The wall-mounted, twelve (12) fiber capacity dual access panels containing SC-type bulkhead connectors and splice trays to be acquired through this RFQ.

RFQ:

Refers to this document known as the Request for Quotation.

UCSB:

Refers to the University of California, Santa Barbara campus, only.

University:

Refers to  the University of California, Santa Barbara campus, only.

The Vendor:

The individual, partnership or corporation, who, as a result of the bidding process, is selected to provide fiber cable termination panels and fiber patch panels under this RFQ.

### 1.5     Quotation Reference Number

All quotations shall reference the RFQ number OW 4816-BD.

### 1.6    Addenda

No one  is authorized to amend any part of this Request For Quotation, in any respect,  by oral statement, or to make representation or interpretation in conflict with its provisions. The University may modify this Request For Quotation, prior to the date of its acceptance, by issuance of written addenda to all parties.

Addenda will clearly be marked as such. Failure of any bidder to receive such addenda or interpretation shall not relieve such bidder from any obligation under their bid as submitted. All addenda so issued shall become part of the Request For Quotation documents.

The date fixed for submission of quotations may be extended, if in the sole judgment of

the University, it is warranted.

### 1.7     Contract Award in Best Interest

The University reserves the right to accept or reject quotations on each item  separately or as a whole, to reject any or all quotations without penalty, to waive informalities

or irregularities, and to contract as the best interest of the University may require in order to obtain the panels which best meet the needs of the University as expressed in this RFQ.

### 1.8     Right to Reject or Modify

Selection of  the winning  Vendor's quotation does  not mean  that all aspects of the quotation(s) are acceptable to the University. The University  retains the right to modify the terms and conditions in the quotation(s) or reject terms and conditions proposed by  the Vendor prior to the execution of the contract as the University, at its sole discretion,

deems necessary to ensure a satisfactory procurement.

### 1.9    Vendors Quotations

To facilitate consideration of their quotations, Vendors must submit three (3) complete copies of all bid documents, in the format described in Section 4 (Vendor Response) of this RFQ.

### 1.10    Contacts With University Personnel

Prior to submission of quotations, it is requested that Vendors coordinate in advance all contacts with University personnel through the University Purchasing Department. Failure to comply with this request may result, during quotation evaluation, in an interpretation of intent to evade RFQ requirements and may subject the Vendor to disqualification.

### 1.11    Warranty

All components and workmanship in panels provided by the selected Vendor shall have a minimum on-site warranty period of one (1) year from the date of receipt of the complete and total number of units ordered as a result of this RFQ.

### 1.12    Defect Notification

The Vendor shall receive detailed  notification of all defective or unacceptable work  from the University, which shall be corrected to the satisfaction of the University's

representative. The corrective actions shall be undertaken by the Vendor within seven (7) calendar days after notification by the University, and shall be completed within  twenty-one (21) calendar days after notification by the University.

### 1.13    Selection Criteria

Quotations will be evaluated by a University Evaluation Team. The contract award, if made, will be made to the Vendor who has the ability to do the following:

#### 1.13.1 Meet all material specifications and requirements of Section 3 of this RFQ,

#### 1.12.2 Meet or exceed the stated schedule for provision of all materials,

#### 1.12.3 Award(s) shall be made to the lowest responsible bidder meeting specifications, or else all quotations shall be rejected. Using this method, all items other than price are specifications that must be met in order for the quotation to be considered.

### 1.14   Basis of Evaluation

Evaluators will review all quotations and assess the degree to which the Vendor's proposal meets the specifications.

The bid shall be awarded to the Vendor meeting all specifications, meeting the stated delivery schedules and submitting the lowest price for all units.

### 1.15   Contract Changes

After selection of the successful bidder, UCSB may contract for additional quantities of fiber termination, building and departmental patch panels at the per unit price quoted in the Vendor's response.

### 1.16    Price Quotation

All quotations for the purchase prices of products stated in the Vendor's quotation shall be fixed from the date on which the quotation is submitted by the Vendor for a period of one hundred and eighty (180) days.

### 1.17    Acceptance Testing Requirements

The Vendor  shall notify  the University, in writing, of the date of completion of all deliveries of items under this RFQ. Inspection and testing of all panels shall be performed by the University's designated representatives.

Acceptance testing shall include, but not be limited to:

#### 1.17.1  Review of  Vendor-provided drawings of fiber termination, building and departmental panel and fiber patch panels.

#### 1.17.2  Inspection of ST bulkhead multi-mode connectors provided within the fiber termination and patch panels for quality of manufacture and mounting alignment.

#### 1.17.3  Inspection of SC bulkhead single mode connectors provided within the fiber termination and patch panels for quality of manufacture and mounting alignment.  (Optional Additive Bid)

#### 1.17.4  Inventory of all components within the panels including splice trays, barrel adapters, entrance grommets, locks, and keys.

#### 1.17.5  Inspection of workmanship as demonstrated in the proper alignment of doors, cable guides, and splice tray mounting,

### 1.18  Schedule for Acquisition

The schedule  for acquisition of the panels described in this RFQ , is as follows:

#### Monday, September 11, 1995: RFQ released for bid

#### Monday, September 25, 1995: Vendor's response to the RFQ due in Purchasing Department.

#### Monday, October 2, 1995: Complete evaluation of RFQ responses, select successful Vendor, and award contract.

#### Monday, October 23, 1995:  Delivery completed for all fiber termination, building and departmental patch panels specified under this RFQ.

The  University may extend these dates, at its sole discretion.

# SECTION 2     BACKGROUND FOR THE QUOTATION

## 2.1  Existing Campus Fiber Cable Plant

The University is building the third and fourth rings of its four ring fiber optic cable plant using forty-eight (48) strand multi-mode fiber strands placed in existing underground ducts to all buildings. Each building location served by the fiber cable will require two (2) fiber termination panels (FTP's), one housing the downstream (incoming) cable and the other housing the upstream (outgoing) cable. All forty-eight fibers in each cable will be terminated within the respective panel.

Fibers rings not being accessed within a building will be continued by placement of short fiber jumper cables between fiber termination panels. Fibers which will support intra-building service will be jumpered to a building patch panel (BPP) using individual

or paired fiber optic jumper cables. Depending upon the number of distinct fiber user organizations within a building, a single or multiple departmental patch panels (DPP's) will be required in each building. These panels have the capacity for either twelve (12) or  twenty-four (24) multi-mode fiber jumpers.

The University's Department of Communications Services functions as a campus utility service providing backbone communications capabilities to every building and department. Individual University Departments may, however, acquire and install their own wiring and equipment within a building. Therefore, the University has specified fiber panel design that allows shared, and controlled, access to each panel for both the utility (Communications Services) and the customer (Building and Department).

## 2.2     Objective of RFQ

The objective  of this  Request for Quotation (RFQ) is to solicit cost quotations and delivery schedules for wall-mounted forty-eight (48) strand multi-mode fiber termination panels (FTP's), wall-mounted twenty-four (24) multi-mode fiber building patch panels (BPP's) and wall-mounted twelve (12) multi-mode fiber departmental (DPP) patch panels. This RFQ specifies panels, enclosed splice trays, ST-type barrel connectors, and cable attachment hardware provided for use of these panels with fiber cables.

The University's design for a fiber cable plant terminates multi-mode fibers on ST-type connectors and attachment of these ST-type connectors to the bulkhead adapters in each fiber termination panel and fiber patch panel.

An Optional Additive Bid Component is defined in Section 2.3.

## 2.3	(Optional Additive Bid)

## Subject to cost quotations supplied by Vendors as a result of this RFQ, the University may choose to acquire additional fiber termination panels for use with a new single-mode fiber optic cable plant. Fiber single-mode termination panels (STP's) for this single-mode installation would use a wall-mounted panel with twelve (12) SC fiber bulkhead adapters. The acquisition of panels for this single-mode installation are defined in this RFQ as an Optional Bid. The University, at its sole discretion,  may choose to purchase, or not purchase, these panels.

# SECTION 3.0     TECHNICAL SPECIFICATIONS AND REQUIREMENTS

The mandatory requirements in this section specify the minimum capabilities required of the components and levels of service to be provided by the Vendor. Prospective Vendors must provide direct, detailed responses to these requirements, in the format specified in Section 4 (Vendor Response) of this RFQ.

## 3.1 Fiber Termination Panels (FTPs)

### 3.1.1. Construction

#### 3.1.1.1.   Forty-eight (48) strand  Fiber Termination Panels (FTP's) shall be complete with splice tray assemblies and ST bulkhead fiber strand connectors equivalent to Telect Part number 015-4331-3322 or AMP 502554-1.

#### 3.1.1.2.   Each FTP unit is wall mounted and supports forty-eight (48) ST -type bulkhead fiber strand connectors.

#### 3.1.1.3.   The outside dimensions of the Fiber Termination Panels shall not exceed sixteen inches by sixteen inches by eight inches (16" x 16" x 8").

#### 3.1.1.4.   The FTP unit is equipped with multiple pre-punched bulkhead connector mounting insert plates.

#### 3.1.1.5.   Each insert plate shall support ST-type connectors spaced to support both simplex or duplex attachment of fiber jumpers using adapters conforming to the specifications of EIA/TIA-568SC.

#### 3.1.1.6.   The FTP unit is equipped with four (4) removable splice trays designed to support twelve (12) strands of fiber per tray for tube breakout and attachment of fan-out protective tubing. Splice tray shall be equivalent to Telect Part number 015-0000-0302.

#### 3.1.1.7.   The FTP unit must be equipped with ground lugs and cable clamps for securing the incoming cable.

#### 3.1.1.8.   The FTP unit includes one-inch (1.0") to two-inch (2.0") wide slots at both the top and bottom left and right sides of the termination panel to allow placement and removal of cables.

#### 3.1.1.9.   The FTP unit contains multiple routing rings for securing the individual fibers in their protective sleeves.

### 3.1.2.  Mounting and Cable Access

#### 3.1.2.1.  University requires that all fiber termination and patch panels be wall-mounted.

#### 3.1.2.2.  No custom brackets or hardware will be considered.

#### 3.1.2.3.  Cable entry and exit from the panels shall be from the top and bottom of the panels in accordance with the required mounting configuration.

#### 3.1.2.4.  Strain relief attachment for the cable strength member shall be included within the box and usable from either top or bottom cable entrance.

#### 3.1.2.5.  Grommet protection for cables at each entrance and exit opening shall be part of the design and provided with the panels.

### 3.1.3.  Splice Trays

#### 3.1.3.1.  A single splice tray shall be used to break-out each bundle of twelve (12) fiber strands and place a protective sheath on each strand. The protected fiber will then be terminated on an ST-type connector.

#### 3.1.3.2.  Four (4)  splice trays shall be provided with each fiber termination panel (FTP). Splice trays shall be equivalent to AMP 501321-2.

### 3.1.4.  Fiber Bulkhead Connectors

The University has standardized on bulkhead connectors which support the AT&T ST-type multimode fiber connectors.

#### 3.1.4.1. Connection loss of less then +/- .5db total, across two (2) ST strand connectors and bulkhead connectors.

#### 3.1.4.2. A precision stainless steel alignment sleeve.

#### 3.1.4.3. Connection repeatability of one thousand (1,000) re-connections, across both ends of the adapter, with less than 0.1db signal degradation.

#### 3.1.4.4. All ST-type bulkhead connectors shall be of the same model number from the same manufacturer.

#### 3.1.4.5. All ST-type bulkhead connectors shall employ spacing consistent with duplex coupling adaptors as defined in EIA/TIA-568SC.

### 3.1.5.  Security Requirements

The Vendor must provide separate, locked access to two separate compartments of the fiber termination panel. The utility (Communications Services) side of the fiber termination panels (FTP's) will house the fiber splice trays (see Section 3.1.3 above) and access to one side of the ST-bulkhead adapters attached to each fiber connector. The customer side of the fiber termination panel will provide access to the opposite side of the ST bulkhead adapters.

The Vendor shall provide locking mechanisms for both sides of each panel. Each locking mechanism shall be capable of unique keying to be specified by the University upon placement of purchase orders.

## 3.2   Building Fiber Patch Panels (BPP's)

### 3.2.1. Construction

#### 3.2.1.1.   Twenty-four (24) strand Fiber Patch Panels (FPP's) complete with splice tray assemblies and ST bulkhead fiber strand connectors equivalent to Telect Part number 015-4231-3322.

#### 3.2.1.2.   Each FTP unit is wall mounted and  supports twenty-four (24) ST -type bulkhead fiber strand connectors.

#### 3.2.1.3.   The outside dimensions of the Fiber Termination Panels shall be a maximum of sixteen inches by sixteen inches by six inches (16" x 16" x 6").

#### 3.2.1.4.   The FTP unit is equipped with multiple pre-punched bulkhead connector mounting insert plates.

#### 3.2.1.5.   Each insert plate shall support ST-type connectors spaced to support both simplex or duplex attachment of fiber jumpers using adapters conforming to the specifications of EIA/TIA-568SC.

#### 3.2.1.6.   The FTP unit is equipped with two (2) removable splice trays designed to support twelve (12) strands of fiber per tray for tube breakout and attachment of fan-out protective tubing. equivalent to Telect Part number 015-0000-0302.

#### 3.2.1.7.   The FTP unit must be equipped with ground lugs and cable clamps for securing the incoming cable.

#### 3.2.1.8.   The FTP unit includes one inch (1.0") to two-inch (2.0") wide slots at both the top and bottom left and right sides of the termination panel to allow placement and removal of cables.

#### 3.2.1.9.   The FTP unit contains multiple routing rings for securing the individual fibers in their protective sleeves.

### 3.2.2.  Mounting and Cable Access

#### 3.2.2.1.  The University requires that all fiber patch panels be wall-mounted.

#### 3.2.2.2.  No custom brackets or hardware will be considered.

#### 3.2.2.3.  Cable entry and exit from the panels shall be from the top and bottom of the panels in accordance with the required mounting configuration.

#### 3.2.2.4.  Strain relief attachment for the cable strength member shall be included within the box and usable from either top or bottom cable entrance.

#### 3.2.2.5.  Grommet protection for cables at each entrance and exit opening shall be part of the design and provided with the panels.

### 3.2.3.  Splice Trays

#### 3.2.3.1.  Each splice tray will be used to break-out a binder group of twelve (12) fiber strands and place a protective sheath on each strand. The protected fiber will then be terminated on an ST-type connector.

#### 3.2.3.2.  Two (2)  splice trays shall be provided with each fiber building patch panel (BPP). Splice trays shall be equivalent to AMP 501321-2.

### 3.2.4.  Fiber Bulkhead Connectors

The University has standardized on bulkhead connectors which support the AT&T ST-type multimode fiber connectors.

#### 3.2.4.1. Connection loss of less then +/- .5db total, across two (2) ST connectors and bulkhead connectors.

#### 3.2.4.2. A precision stainless steel alignment sleeve.

#### 3.2.4.3. Connection repeatability of one thousand (1,000) re-connections, across both ends of the bulkhead connector, with less than 0.1db signal degradation.

#### 3.2.4.4. All ST-type bulkhead connector shall be of the same model number from the same manufacturer.

#### 3.2.4.5. All ST-type bulkhead connectors shall employ spacing consistent with duplex coupling adaptors as defined in EIA/TIA-568SC.

### 3.2.5.  Security Requirements

The Vendor must provide separate, locked access to two separate compartments of the building fiber optic patch panels. The utility (Communications Services) side of the Building fiber patch panels (BPP's) will house the fiber splice trays (see Section 3.2.3 above) and access to one side of the ST-bulkhead adapters attached to each fiber connector. The customer side of the fiber building  patch panels will provide access to the opposite side of the ST bulkhead adapters.

The Vendor shall provide locking mechanisms for both sides of each panel. Each locking mechanism shall be capable of unique keying to be specified by the University upon placement of purchase orders.

## 3.3   Department Fiber Patch Panels (DPP's)

### 3.3.1. Construction

#### 3.3.1.1.   Twelve (12) strand fiber Department Patch Panels (DPP's) ST bulkhead fiber strand connectors shall be equivalent to Telect Part number 015-4122-0000.

#### 3.3.1.2.   Each DPP unit is wall mounted and supports twelve (12) ST -type bulkhead fiber connectors.

#### 3.3.1.3.   The outside dimensions of the Department Patch Panels are a maximum of sixteen inches by twelve inches by four inches (16" x 12" x 4").

#### 3.3.1.4.   The FTP unit is equipped with multiple pre-punched bulkhead connector mounting insert plates.

#### 3.3.1.5.   Each insert plate shall support ST-type connectors spaced to support both simplex or duplex attachment of fiber jumpers using adapters conforming to the specifications of EIA/TIA-568SC.

#### 3.3.1.6.  The FTP unit must be equipped with ground lugs and cable clamps for securing the incoming cable.

#### 3.3.1.7.   The FTP unit includes one-inch (1.0") to two-inch (2.0") wide slot at both the top and bottom left and right sides of the termination panel to allow placement and removal of cables.

#### 3.3.1.8.   The FTP unit contains multiple routing rings for securing the individual fibers in their protective sleeves.

### 3.3.2.  Mounting and Cable Access

#### 3.3.2.1.  University requires that all fiber optic patch panels be wall-mounted.

#### 3.3.2.2.  No custom brackets or hardware will be considered.

#### 3.3.2.3.  Cable entry and exit from the panels shall be from the top and bottom of the panels in accordance with the required mounting configuration.

#### 3.3.2.4.  Strain relief attachment for the cable strength member shall be included within the box and usable from either top or bottom cable entrance.

#### 3.3.2.5.  Grommet protection for cables at each entrance and exit opening shall be part of the design and provided with the panels.

### 3.3.3.  Splice Trays

#### Splice trays are not required for the Departmental Patch Panels (DPP's).

### 3.3.4.  Fiber Bulkhead Connectors

The University has standardized on bulkhead connectors which support the AT&T ST-type multimode fiber connectors.

#### 3.3.4.1. Connection loss of less then +/- .5db total, across two (2) ST connectors and bulkhead connectors.

#### 3.3.4.2. A precision stainless steel alignment sleeve.

#### 3.3.4.3. Connection repeatability of one thousand (1,000) re-connections, across both ends of the connector, with less than 0.1db signal degradation.

#### 3.3.4.4. All ST-type bulkhead connectors shall be of the same model number from the same manufacturer.

#### 3.3.4.5. All ST-type bulkhead connectors shall employ spacing consistent with duplex coupling adaptors as defined in EIA/TIA-568SC.

### 3.3.5  Security Requirements

The Vendor must provide separate, locked access to two separate compartments of the fiber optic departmental patch panels. The utility (Communications Services) side of the fiber optic departmental patch panels (DPP's) will house the fiber splice trays (see Section 3.3.3 above) and access to one side of the ST-bulkhead connectors attached to each fiber connector. The customer side of the fiber optic departmental patch panel will provide access to the opposite side of the ST bulkhead connectors.

The Vendor shall provide locking mechanisms for both sides of each panel. Each locking mechanism shall be capable of unique keying to be specified by the University upon placement of purchase orders.

## 3.4  Single Mode Fiber Termination Panels (STP's)  (Optional Additive Bid)

### 3.4.1. Construction

3.4.1.1.   Twelve (12) strand Single Mode Fiber Termination Panels (STP's) complete with splice tray assemblies and 568SC-type bulkhead fiber connectors.

#### 3.4.1.2.   Each STP unit is wall mounted and supports twelve (12) 568SC-type bulkhead fiber strand connectors.

#### 3.4.1.3.   The outside maximum dimensions of the Single Mode Termination Panels are sixteen inches by sixteen inches by four inches (16" x 16" x 4").

#### 3.4.1.4.   The STP unit is equipped with a removable splice trays designed to support twelve (12) strands of fiber per tray for tube breakout and attachment of fan-out protective tubing.

#### 3.4.1.5.   The STP unit must be equipped with ground lugs and cable clamps for securing the incoming cable.

#### 3.4.1.6.   The STP unit includes one inch (1.0") to two-inch (2.0") wide slots at both the top and bottom left and right sides of the termination panel to allow placement and removal of cables.

#### 3.4.1.7.   The STP unit contains multiple routing rings for securing the individual fibers in their protective sleeves.

### 3.4.2.  Mounting and Cable Access

#### 3.4.2.1.  University requires that all fiber optic termination panels be wall-mounted.

#### 3.4.2.2.  No custom brackets or hardware shall be required for mounting.

#### 3.4.2.3.  Cable entry and exit from the panels shall be from the top and bottom of the panels in accordance with the required mounting configuration.

#### 3.4.2.4.  Strain relief attachment for the cable strength member shall be included within the box and usable from either top or bottom cable entrance.

#### 3.4.2.5.  Grommet protection for cables at each entrance and exit opening shall be part of the design and provided with the panels.

### 3.4.3.  Splice Trays

#### 3.4.3.1.  A single splice tray to break-out the twelve (12) fiber strands and place a protective sheath on each strand. The protected fiber will then be terminated on an 568SC-type connector.

#### 3.4.3.2.  A splice tray shall be provided with each fiber single mode termination panel (STP).

#### 3.4.3.3.  Splice trays shall be equivalent to AMP 501321-2.

### 3.4.4.  Fiber Bulkhead Connectors  (Optional)

The University will use 568SC-type single-mode fiber connectors.

#### 3.4.4.1  All single-mode bulkhead connectors proposed shall meet the specifications defined by TIA/EIA 568 for 568SC single-mode connectors.

#### 3.4.4.2. Connectors shall use a precision stainless steel alignment sleeve.

#### 3.4.4.3. Maximum optical attenuation per each mated field installed 568SC connector pair shall not exceed 0.75db as measured by ANSI/EIA/TIA-455-59 (field test method).

#### 3.4.4.4  Each connector shall have a return loss less than or equal to 26db on single-mode fiber per ANSI/EIA/TIA-455-8 (field test method).

#### 3.4.4.5. Connection repeatability of five hundred (500) re-connections without violation of TIA/EIA-568SC performance specifications.

#### 3.4.4.6. All 568SC-type bulkhead adapters shall be of the same model number from the same manufacturer.

#### 3.4.4.7. The bulkhead connector mounting shall support horizontal duplex fiber attachment per implementation of EIA/TIA-568SC.

### 3.4.5  Compatibility With Adapter for Joining /Alignment of 568SC Fiber Connectors:

#### 3.4.5.1. All 568SC connectors shall be fully compatible with a 568SC adapter as defined by EIA/TIA-568SC.

#### 3.4.5.2. The 568SC adapter shall be either two (2) simplex 568SC connectors or a duplex SC adapter of one piece construction.

#### 3.4.5.3. The adapter shall maintain nominal 12.7 mm (0.5 in) center spacing in the optical fiber patch panel.

#### 3.4.5.4. The adaptor shall meet the dimension requirements, including the key and keyways as defined in ANSI/EIA/TIA-604-3.

#### 3.4.55. The adapter shall support horizontal mounting of the 568SC connectors within the optical fiber patch panel.

#### 3.4.5.5. Keying in the 568SC adapter shall be labeled and referred to as Position A and Position B.

### 3.4.6.  Security Requirements

The Vendor must provide separate, locked access to two separate compartments of the fiber termination, building and departmental fiber patch panels. The utility (Communications Services) side of the fiber single mode termination panels (STP's)

will house the fiber splice trays (see Section 3.4.3 above) and access to one side of the 568SC-type bulkhead adapters attached to each fiber connector. The customer side of the fiber termination panels will provide access to the opposite side of the 568SC-type bulkhead adapters.

# The Vendor shall provide locking mechanisms for both sides of each panel. Each locking mechanism shall be capable of unique keying to be specified by the University upon placement of purchase orders.

# SECTION 4.0    VENDOR RESPONSE

## 4.1  Vendor Information

### Vendor shall provide the following information as their response to this RFQ.

### 4.1.1  Vendor Name: ______________________________

### 4.1.2  Vendor representative preparing this RFQ:________________________

### 4.1.3  Vendor Mailing Address: ____________________________________

### ____________________________________

### ____________________________________

### 4.1.4  Vendor Representative Phone Number:   ___________________

### 4.1.5  Vendor shall use the pricing submittal sheet ONLY for their responses.

### 4.1.6  All pricing shall include applicable sales tax.

### 4.1.7  Vendor shall state shipping and handling cost separately.

## 4.2  Pricing

### 4.2.1   Forty (40) forty-eight (48) strand multi-mode fiber termination panels (FTP's).

#### Total Cost Forty (40) 48-Fiber FTP's  $ ______________________________

#### Shipping and handling costs for above $ ________________

### 4.2.2   The individual, per panel, cost for additional fiber termination panels (FTP's) ordered individually.

#### Unit Cost 48-Fiber FTP's  $ _______________________per unit

#### Shipping and handling costs for above $ ________________

### 4.2.3   Twenty (20) twenty four (24) strand multi-mode fiber building patch panels (BPP's).

#### Total Cost Twenty (20) 24-Fiber BPP's  $ ______________________________

#### Shipping and handling costs for above $ ________________

### 4.2.4   The individual, per panel, cost for additional building patch panels (BPP's) ordered individually.

#### Unit Cost 24-Fiber BPP's  $ ______________________________

#### Shipping and handling costs for above $ ________________

### 4.2.5   Twenty (20) twelve (12) strand multi-mode fiber department patch panels (DPP's).

#### Total Cost twenty (20) 12-Fiber DPP's  $ ______________________________

#### Shipping and handling costs for above $ ________________

### 4.2.6   The individual, per panel, cost for additional department patch panels (DPP's).

#### Unit Cost 12 Fiber DPP's  $ ______________________________

#### Shipping and handling costs for above $ ________________

### 4.2.7   Forty (40) twelve (12) strand single mode fiber termination panels (STP's) ordered individually. (Optional Additive Bid)

#### Total Cost forty (40) Single Mode Fiber STP's $__________________________

#### Shipping and handling costs for above $ ________________

### 4.2.8   The individual, per panel, cost for additional single mode fiber termination panels (STP's) ordered individually. (Optional Additive Bid)

#### Unit Cost Single Mode Fiber STP's  $ ______________________________

#### Shipping and handling costs for above $ ________________

## 4.3  Specifications

## The Vendor shall provide the following information and samples as part of their response to this bid:

### 4.3.1 Ordering Part Numbers - Multi-mode Fiber Terminal Panels (FTP's)

### Ordering Part # ___________________________

### Manufacturer Part # ___________________________

### 4.3.2 Ordering Part Numbers - Multi-mode Fiber Building Patch Panels (BPP's)

### Ordering Part # ___________________________

### Manufacturer Part # ___________________________

### 4.3.3 Ordering Part Numbers -Multi-mode Fiber Department Patch Panels (DPP's)

### Ordering Part # ___________________________

### Manufacturer Part # ___________________________

### 4.3.4 Ordering Part Numbers - Single mode Fiber Termination Panels (STP's)

### (Optional Additive Bid)

### Ordering Part # ___________________________

### Manufacturer Part # ___________________________

### 4.3.5  Vendor shall provide a separate detailed drawing for the fiber termination panel (FTP), fiber building patch panel (BPP), fiber departmental patch panel (DPP) and (optional bid) single mode fiber patch panels (STP's).

### 4.3.6  Vendor shall provide a copy of the manufacturer's specification and description sheet for each fiber termination panel (FTP), fiber building patch panel (BPP), fiber departmental patch panel (DPP) and (optional bid) single mode fiber patch panels (STP's).

### 4.3.7. Vendor shall provide two (2) samples of the ST-type barrel connector to be supplied with the fiber termination and patch panels.

### 4.3.8  Vendor shall provide two (2) samples of a duplex multimode adapter for ST-type connectors compatible with the proposed fiber termination and patch panels.

### 4.3.9  Vendor shall provide two (2) samples of the duplex singlemode adaptor for 568SC-style connectors compatible with the proposed fiber termination and patch panels.

## 4.4   Delivery

## Delivery of all fiber termination and patch panels purchased as a result of this RFQ should be made no later than twenty-one (21) days after issuance of a University Purchase Order. Vendor shall confirm the availability of fiber termination and patch panels defined in Sections 4.3.1, 4.3.2, 4.3.3 and 4.3.4 (above).

### 4.4.1 Delivery of the multi-mode fiber termination panels (Section 3.1) can be made no later than __________ days following issuance of a University Purchase Order.

#### ___________________________________________

#### Signature					Date

### 4.4.2  Delivery of the multi-mode fiber building patch panels (Section 3.2)  shall be made no later than  __________ days following issuance of a University Purchase Order.

#### ___________________________________________

#### Signature					Date

### 4.4.3  Delivery of the multi-mode fiber departmental patch  panels (Section 3.3)  shall be made no later than ___________ days following issuance of a University Purchase Order.

#### ___________________________________________

#### Signature					Date

### 4.4.4  Delivery of the single mode fiber termination panels (Section 3.4)  shall be made no later than ____________ days following issuance of a University Purchase Order.

#### ___________________________________________

#### Signature					Date

## 4.5  Warranty

### 4.5.1   Vendor shall provide a minimum one-year warranty on all parts and workmanship for all termination and patch panels purchased as a result of the RFQ. Warranty period shall begin from the date or receipt of all termination and patch panels ordered as a result of this RFQ.

### 4.5.2   A statement of the Vendor's warranty for fiber termination panels (FTPs), building patch panels (BPPs), departmental patch panels (DPP) and single mode fiber termination panels (STPs) shall be provided.