Request for Proposal (RFP) for Dial-Up Modem and ISDN Services to the Internet and UCSB Campus Network

Issued by the University of California, Santa Barbara Campus

µ

1.0	 Purchasing Letter	4

1.1.	Purpose	4

1.2.	Issuing Office	4

1.3.	Terms and Conditions	4

1.4.	Definitions	4

1.5.	Proposal Reference Number	5

1.6.	Addenda	5

1.7.	Contract Award in Best Interest	5

1.8.	Right to Reject or Modify	5

1.9.	Vendor qualifications	6

1.10.	Vendors Proposals	6

1.11.	Warranty	7

1.12.	Defect Notification	7

1.13.	Evaluation Criteria/Basic for Award	7

1.14.	Presentation	8

1.15.	Marketing	8

1.16.	Promotional Activities	9

1.17.	Price Quotation	9

1.18.	Pricing and Contract Term	9

1.19.	Schedule for RFP Proposal and Delivery	9

1.20.	University contacts:	10

2.0	Specifications	11

2.1	Introduction	11

2.2	Required Features of Dial-up Modem Service	12

2.2.1.	Support for Industry Standard Modems.	12

2.2.2.	Transport of Protocols.	12

2.2.3.	Local Dial-Up Access.	12

2.2.4.	Access to the UCSB Campus Network.	13

2.2.5.	Access to the Internet.	13

2.2.6.	System Topology Diagrams	13

2.2.7.	Subscriber Billing Procedures.	14

2.2.8.	Responsibility for Equipment and Software.	14

2.2.9.	University Responsibilities	15

2.2.10.	Subscriber help desk services.	16

2.2.11.	Location of Vendor's Equipment	17

2.2.12.	System Reliability	20

2.2.13.	System Maintenance	20

2.2.14.	Internet Registration	21

2.2.15.	Security Procedures and Authentication Methods	21

2.2.16.	Test Accounts	21

2.2.17.	System Capacity	21

2.2.18.	Tiers of Service	22

2.3	Desirable Features of Dial-up Modem Service	23

2.3.1	Multicast Support	23

2.3.2	Internet Services	23

2.3.3	Client Application Software	23

2.3.4	Client Application Software Help Desk Services	23

2.4	Required Features of ISDN Service	24

2.4.1.	ISDN Line Rates and Channel Bonding (inverse multiplexing)	24

2.4.2.	Transport of Protocols.	24

2.4.3.	Local Dial-Up Access.	24

2.4.4.	Access to the UCSB Campus Network.	25

2.4.5.	Access to the Internet.	25

2.4.6.	System Topology Diagrams	25

2.4.7.	Subscriber Billing Procedures.	25

2.4.8.	Responsibility for Equipment and Software.	26

2.4.9.	University Responsibilities	27

2.4.10.	Subscriber help desk services.	27

2.4.11.	Location of Vendor's Equipment	29

2.4.12.	System Reliability	31

2.4.13.	System Maintenance	32

2.4.14.	Internet Registration	33

2.4.15.	Security Procedures and Authentication Methods	33

2.4.16.	Test Accounts	33

2.4.17.	System Capacity	33

2.4.18.	Tiers of Service	34

2.5	Desirable Features of ISDN Service	35

2.5.1.	Residential Wiring Services.	35

2.5.2.	Internet Services	35

2.5.3.	Client Application Software	35

2.5.4.	Client Application Software Help Desk Services	35

2.5.5.	Vendor-provided Hardware	35

2.5.6.	Local Exchange Area ISDN Usage Charges.	36

3.0	Vendor's Response Forms	37

Table 1: Usage Statistics for Existing Pool of 128 UCSB Public Modems	42

Table 2: Estimated Faculty, Staff and Student Residences, by Zip Code	43

# 1.0	 Purchasing Letter

## 1.1.	Purpose

The purpose of this Request For Proposal (RFP) is for vendors to provide the Regents of the University of California, Santa Barbara campus (UCSB), with the specifications, performance schedule and price quotations for specific dial-up modem and/or ISDN service connections to the Internet and UCSB campus network, as described in Section 2.

## 1.2.	Issuing Office

This RFP is issued for the University by the Purchasing Department.  Proposals must be received by 4:30 p.m. on June 28, 1996 and submitted in eight (8) copies to:

By US Postal Service: 	By UPS, FedEx, or hand delivered:

University of California, Santa Barbara	University of California, Santa Barbara

Purchasing Department	Purchasing Department

Santa Barbara, CA 93106-1150	41 Aero Camino

Goleta, CA 93117

Proposals will not be accepted via facsimile transmission.

## 1.3.	Terms and Conditions

The terms and conditions specified in Appendix A shall apply.  University of California standard purchase order terms and conditions shall apply to any order which may result from vendor's offer.  If the vendor includes supplemental information on their quotation form, any terms and conditions imprinted on that form are understood not to apply.

## 1.4.	Definitions

For the sake of brevity, the following definitions should be used whenever the following  terms are used throughout the RFP:

Downtime:

The Vendor's service will be considered "down" if it is inaccessible to all subscribers due to scheduled maintenance, if it is inaccessible to all subscribers due to unscheduled problems, if there is a performance problem which is providing degraded service to all subscribers, or if there is a performance problems which is preventing service to as many as five percent (5%) of the subscribers.

Prime Time:

In the context of providing dial-up services (via either modem or ISDN connections), the hours of noon to midnight each day of Monday through Friday shall be considered prime time hours.  The hours outside of these times shall be considered non-prime time hours.

Subscriber:

Refers to an individual faculty, staff or student customer of the vendor's services.

UCSB:

Refers to the University of California, Santa Barbara campus, only.

University:

Refers to the University of California, Santa Barbara campus, only.

Vendor:

Any individual, partnership or corporation who participates in the RFP process, prior to award.  After award, the successful vendor shall be the individual, partnership or corporation, who, as a result of the RFP process, is selected to provide the services.

## 1.5.	Proposal Reference Number

All quotations shall reference the RFP number, SX-3128.

## 1.6.	Addenda

No one is authorized to amend any part of this Request For Proposal, in any respect, by oral statement, or to make representation or interpretation in conflict with its provisions.  The University may modify this Request For Proposal, prior to the date of its acceptance, by issuance of written addenda to all parties who have received the original RFP.  The date fixed for submission of proposals may be extended, if, in the sole judgment of the University, it is warranted.  All addenda will clearly be marked as such.  Failure of any qualified vendor to receive such addenda or interpretation shall not relieve such vendor from any obligation under their proposal as submitted.  All addenda so issued shall become part of the Request For Proposal documents.

## 1.7.	Contract Award in Best Interest

The University reserves the right to accept or reject proposals on each item separately or as a whole, to reject any or all proposals without penalty, to waive informalities or irregularities, and to contract as the best interest of the University may require in order to obtain the services/materials which best meets the needs of the University as expressed in this RFP.

## 1.8.	Right to Reject or Modify

Selection of the winning vendor's proposal does not mean that all aspects of the proposal are acceptable to the University.  The University retains the right to modify the terms and conditions in the proposal or reject terms and conditions proposed by the vendor prior to the execution of the contract as the University, at its sole discretion, deems necessary to ensure a satisfactory procurement.  The University may, by written notice stating the extent and effective date, cancel and/or terminate any order resulting from this solicitation for cause in whole or in part, at any time.

## 1.9.	Vendor qualifications

The University considers it to be of greatest importance that the successful vendor present evidence of their ability to perform prior to the awarding of a Contract by the University.  No contract will be awarded to a vendor where the dollar amount of such a contract would represent more than twenty five percent (25%) of the vendor's gross yearly income.  To have a proposal considered, the vendor must be well qualified in the following categories:

1.	Demonstrated experience in furnishing dial-in modem services and access to the Internet, based on the technical requirements stated in section 2.

2.	Experience and reputation in the college and university market, preferably with a track record of performance on similar contracts, including demonstrated high quality customer service, as evaluated from comments of vendor references.

Vendors must attach a list of three (3) references of evidence of their track records and performance of similar contracts.  References will preferably be for dial-in modem, ISDN and Internet services provided to colleges or universities.  Customer references should include the customer's (university's or college's) name, address and telephone number, name of a contact and dates of service performance.  In addition, list any contracts that have been canceled or terminated and the reasons for such cancellation or termination.

## 1.10.	Vendors Proposals

In order to be considered, vendors must submit a complete response to this RFP following the outline of the requirements provided.  Distribution of the proposals to parties other than those specifically designated is subject to University approval.  Failure to observe this requirement may result in vendor disqualification.

To facilitate consideration of their proposals, vendors are to submit eight (8) complete copies of all bid documents, in the format described below.

Space has been provided in this document for vendors to supply price quotations, responses to specific questions, and comments.  The space appears at the end of each sub-section within Sections 2.0 and 3.0.   Supporting explanatory information may be submitted and will be reviewed.  To promote compatibility among proposals and to ease the evaluation process, your response to this request MUST be written or typed on the RFP in the space provided in Sections 2.0 and 3.0, or on separate sheets that exactly match the format of Sections 2.0 and 3.0; supporting information should be attached and annotated with the section to which it supports.  Vendors must sign the bid on the last page of the RFP.

Failure to follow these requirements may result in disqualification.

## 1.11.	Warranty

All services/materials provided by the selected vendor shall have a minimum on-site warranty period of one (1) year from the date specified in the written Notice of Final Acceptance.

## 1.12.	Defect Notification

The vendor shall receive detailed notification of all defective or unacceptable services/materials from the University, which shall be replaced to the satisfaction of the University's representative.  The replacement shall be made by the vendor within thirty (30) days after notification by the University.

## 1.13.	Evaluation Criteria/Basic for Award

The University will accept and evaluate proposals from Vendors that can provide dial-up modem services or ISDN services.  While it is preferable to make a single award for all services, the University will award each service independently to the lowest responsible bidder that meets all mandatory requirements of this RFP for that specific service.

Proposals will be evaluated by a University Evaluation Team. The contract award, if made, will be made on the basis of a "cost per quality point" method.  An average of all quality points awarded by individual evaluators will be computed.  The cost of the services/materials will be divided by the total quality points.  The bidder with the lowest cost per quality point will be awarded the contract, based on the following criteria:

a.	Responsible and responsive proposal meeting or exceeding specifications in all sections of the RFP document, including submission of data requested.

b.	Ability to deliver services/materials in the time period requested.

c.	Ability to provide responsive and reliable services/materials.

d.	Demonstrated experience in providing same or similar services/materials.

e.	Verifiable references.

f.	Warranty and period.

The lowest bid price for each service will be determined by the following formulas, which result in a weighted total dollar value per hour of prime time service for 1,000 customers in their first month of service.  Each formula reflects University service considerations and weighting criteria for each particular service.

Cost of Dial-in Modem service for bid evaluation purposes =

(700)*(Q1/40+Q2/40) + (200)*(Q3/60+Q4/60) + (100)*(Q5/264+Q6/264)

Cost of ISDN service for bid evaluation purposes =

(700)*(Q7/60+Q8/60) + (200)*(Q9/60+Q10/60) + (100)*(Q11/264+Q12/264)

Where:

Q1 =	Quote for one-time setup charge for "entry" subscriber level of dial-in modem service (Section 3.5.1.1)

Q2 = 	Quote for monthly charge for "entry" subscriber level of dial-in modem service (Section 3.5.1.1)

Q3 =	Quote for one-time setup charge for "mid" subscriber level of dial-in modem service (Section 3.5.1.2)

Q4 =	Quote for monthly charge for "mid" subscriber level of dial-in modem service (Section 3.5.1.2)

Q5 =	Quote for one-time setup charge for "premier" subscriber level of dial-in modem service (Section 3.5.1.3)

Q6 =	Quote for monthly charge for "premier" subscriber level of dial-in modem service (Section 3.5.1.3)

Q7 =	Quote for one-time setup charge for "entry" subscriber level of ISDN service (Section 3.5.2.1)

Q8 = 	Quote for monthly charge for "entry" subscriber level of ISDN service (Section 3.5.2.1)

Q9 =	Quote for one-time setup charge for "mid" subscriber level of ISDN service (Section 3.5.2.2)

Q10 =	Quote for monthly charge for "mid" subscriber level of ISDN service (Section 3.5.2.2)

Q11 =	Quote for one-time setup charge for "premier" subscriber level of ISDN service (Section 3.5.2.3)

Q12 =	Quote for monthly charge for "premier" subscriber level of ISDN service (Section 3.5.2.3)

The above formulas assume that 70% of the 1,000 subscribers select the "entry" tier, 20% select the "mid" tier and 10% select the "premier" tier.  These assumptions are being made for proposal evaluation purposes only, and University does not guarantee that subscribers will select tiers of service in these proportions.

## 1.14.	Presentation

Vendors who receive at least seventy percent (70%) of the total quality points will be selected for final evaluation.  Vendors selected for final evaluation may be required to make an oral presentation of their proposal.  Such presentations provide an opportunity for the vendor to clarify the proposal to ensure thorough mutual understanding.  Scheduling of the time and location for these presentations will be arranged by the University Communications Services Department.  Failure to comply with this scheduling procedure may result in vendor disqualification.

## 1.15.	Marketing

University shall, in accordance with University's policies and procedures, assist and cooperate with Vendor in the marketing and promotion of Vendor's services to subscribers.  Such assistance shall include but not be limited to providing: access to high traffic areas for booth and tent promotions; access to student bulletin boards for posters and "take-ones"; and access to student orientation activities.  University shall provide Vendor with the name it desires to use to describe the marketing campaign for Vendor's services.  University hereby consents to Vendor's use of University's logo, trademarks and service marks in marketing services to subscribers.

## 1.16.	Promotional Activities

Subject to the prior written consent of University, which consent shall not be unreasonably withheld, Vendor may, on its own behalf or through third party agents, from time to time conduct on-campus promotions and/or publicity of Vendor's services.  University shall assist Vendor or its agents, in accordance with University's policies and procedures, in facilitating such promotions and/or publicity.

## 1.17.	Price Quotation

All quotations for prices of products or services stated in the vendor's proposal shall be fixed from the date on which the proposal is submitted by the vendor for a period of sixty (60) days.  The University is not liable for any cost incurred by prospective vendors prior to issuance of award.   All prices to be quoted F.O.B. Destination.

## 1.18.	Pricing and Contract Term

This contract will be for an initial period of one (1) year, commencing on the date of final acceptance of the service into production use by University, with up to three (3) annual renewal options thereafter for up to four (4) years total, at the University's option.  University will receive the benefit of any vendor price decreases within the contract period.  Proposals for vendor rate changes for affected services shall be submitted a minimum of 120 days in advance of contract renewals.  Rate changes must be approved by the University prior to implementation and shall remain firm for the following one (1) year renewal period.

In the event rate changes are not approved by the University, the University shall provide the Vendor a one-hundred-twenty (120) day notice of intent to cancel.  During the one-hundred-twenty (120) day notice period there can be no change in rates or service levels and the Vendor shall honor any and all orders received prior to the completion of the one-hundred-twenty (120) day notice period.

## 1.19.	Schedule for RFP Proposal and Delivery

The schedule for this RFP is as follows.  The University may extend these dates, at its sole discretion.

May 17, 1996 :	RFP issued by University Purchasing Department.

June 14, 1996 :	Last date on which written questions will be accepted.

June 28, 1996 :	Vendor's response to the RFP due in Purchasing Department.

July 19, 1996 :	Complete evaluation of RFP responses and select successful vendor.

September 20, 1996 :	Preferred commencement date of vendor's services.

## 1.20.	University contacts:

All questions about the contents of this RFP should be submitted in writing to one of the following two contacts:

Purchasing/Administrative issues:  	Daily Baise

Purchasing Department

University of California

Santa Barbara, CA 93106

(805) 893-2093

Technical/Specification issues:	Vince Sefcik

Communications Services

University of California

Santa Barbara, CA

(805) 893-4182

electronic mail address: com1vs@ucsbvm.ucsb.edu

# 2.0	Specifications

# 2.1	Introduction

The University of California, Santa Barbara campus (UCSB), is a public research institution with an enrollment of approximately 17,800 students and 8,300 faculty and staff.  As is prevalent in most institutions of higher learning, UCSB has relied heavily upon electronic communications to fulfill its missions of research, education and public service.  There has been a recent change, however, with respect to the electronic communications needs of faculty, student and staff access outside of the traditional classroom or office setting.  This change has been precipitated by the explosive growth in the number of home computers owned by members of the campus community, the decrease in modem costs, an increased trend towards telecommuting among faculty and staff, and the growth of national and international information resources available for access via the Internet.

In response to the increased demand for off-campus electronic communication with the campus host computers and the Internet, the Communications Services department has increased the number of modems available to the campus community to the present size of 128 modems.  The use of these modems has been provided at no charge to the individual campus members.  In addition, several other campus departments provide their own departmental modem pools.  Even with growth in the number and the size of the modem pools over the past few years, demand for modem connections continues to increase beyond the university's ability to fund this service.  Therefore, the university has decided to outsource the future growth this service.

UCSB requests proposals from communications service providers for dial-up modem and ISDN services to the Internet and to the UCSB campus network.  While services are desired to be state of the art, backward compatibility with older modems is desired as well.  The greatest demand for these services is expected to be generated in the areas surrounding (60 mile radius) the campus location.  In addition to these primary locations, services also may be used by University faculty, staff, and students outside a 60 mile radius when traveling throughout the state or the nation.

To provide information on potential volumes of services, the following chart presents the number of employees, students, and current number of public modems at UCSB.  The employee and student figures are based upon Fall 1994 data and are rounded to the nearest 100.  The number of modems is based upon the most recent data available.  While it is not possible for the University to estimate the eventual volume of modem use, it has provided usage statistics for the existing public modem pool in Table 1.  Vendors should note that these statistics were generated based upon the availability of modem services at no cost to the caller.

No. of Employees	No. of Students	Total	No. of modems

8,300	17,800	26,100	128

Even though the University intends to retain its existing modem pools in service, Vendors should be prepared for a large increase in dial-up service demand and be prepared to scale services to respond.

The University will accept and evaluate proposals from Vendors that can provide dial-up modem services or ISDN services.  While it is preferable to make a single award for all services, the University will award each service independently to the lowest responsible bidder that meets all mandatory requires of this RFP for that specific service.

# 2.2	Required Features of Dial-up Modem Service

### 2.2.1.	Support for Industry Standard Modems.

A full range of industry standard modems must be supported, including v.22, v.32, v.32bis, v.42bis, v.34 and v.FC, with modulation and error recovery at communications rates from 2,400 bits per second (bps) to 28,800 bps (28.8 Kbps), with all connections to Vendor's equipment capable of operating at 28.8 Kbps simultaneously.

Comply?  YES ____   NO ____

### 2.2.2.	Transport of Protocols.

A broad range of subscriber computer communications software is currently in use.  Line protocols that must be supported by the vendor's services include SLIP, CSLIP, PPP and character based terminal sessions.  Higher level protocols that must be transported by the vendor's services include TCP, IP and UDP.  Transport of all standard application protocols, i.e., File Transfer Protocol (FTP), terminal to host connections (telnet, tn3270), electronic mail (SMTP, POP-2, POP-3), Domain Name Services (DNS), Directory Services (PH, Whois, finger), Gopher, Boot Protocol (BootP), NNTP News Services and World Wide Web (WWW) must be supported.

Comply?  YES ____   NO ____

### 2.2.3.	Local Dial-Up Access.

Subscribers of the dial-up services should not be required to call outside of a local calling zone or make a long-distance call to connect to the vendor's facilities.  The majority of potential subscribers reside within the same GTE Local Calling Area as the campus.  However, a significant portion of subscribers reside within the Local Access Transport Area (LATA), but outside of the GTE Local Calling Area (e.g., residents in Lompoc, Santa Maria and Carpinteria).  See Table 2 for data on where faculty, staff and students live, by Zip Code.  Vendors must specify how subscribers within the LATA but outside of the Local Calling Area will access the vendor's services without paying charges for long distance calls (e.g., by use of multiple Points of Presence, use of Foreign Exchange lines, use of a toll-free 800 number, etc.).

Comply?  YES ____   NO ____

Explanation of how requirement will be met:

### 2.2.4.	Access to the UCSB Campus Network.

Communications between the vendor's service and the UCSB network must use the IP protocol.  If the vendor's equipment is collocated at the UCSB campus, connection of the vendor's equipment to the UCSB campus network will be via an Ethernet multiport transceiver provided by UCSB.  If the vendor's equipment is located off-campus, connection of the vendor's equipment to the UCSB campus network will be via a communications circuit and an existing router installed on the UCSB campus.  The physical communications circuit between the Vendor's equipment and the UCSB equipment may be a dedicated circuit or Frame Relay service.  The Vendor is responsible for all costs to install, operate and maintain the communications circuit between the Vendor's equipment and the UCSB equipment.

Comply?  YES ____   NO ____

### 2.2.5.	Access to the Internet.

Connectivity between the vendor's service and the Internet must not traverse the UCSB campus network.  That is, the vendor must provide a dedicated connection from their service directly to the Internet for all communications with hosts other than hosts on the UCSB network.

Comply?  YES ____   NO ____

### 2.2.6.	System Topology Diagrams

Vendor shall provide the University with system topology diagrams.  The diagrams shall include information on both the dial-in modem equipment and software, and the Vendor's connection to the Internet.  The topological diagram of the connection to the Internet must show the path from the Vendor's local equipment to each of the national Network Access Points (NAPs), including redundant paths and redundant equipment.

Comply?  YES ____   NO ____

Attach topological diagrams of your network's connection to the Internet and paths to each of the national NAPs.

### 2.2.7.	Subscriber Billing Procedures.

Each subscriber will select the billing procedures to be used for their subscription. At a minimum, the Vendor must provide 2 billing options:

#### 2.2.7.1	Issue a separate invoice to the subscriber at his/her residence for the Vendor's services, and

#### 2.2.7.2	Bill a University department, school, or college, or an office of the central campus administration.  Selection of this billing option by a subscriber must require prior approval by a University employee with the appropriate funding authority.  The method for securing prior approval by a University employee shall be mutually developed and agreed upon by the Vendor and the University.  All expenses invoiced to the University will be in the form of a "composite" invoice (not individual invoices for each subscriber) and the subscriber-specific invoice detail must be provided in both printed and computer-readable (e.g., magnetic tape or disk) format.  The invoice must provide sufficient detail to allow the recipient department to assign costs to a specific individual.  The Vendor shall provide detailed information on the format of the composite computer-readable invoice.

Each subscriber must be able to individually select one of these two minimum billing options.  In addition to these two options, Vendors may propose adding charges to the subscriber's current residence telephone line invoice via arrangements with the local telephone company (GTE).  Vendors may propose other options for billing in addition to the above, as long as they satisfy the minimum billing options.

Comply?  YES ____   NO ____

Attach a sample invoice to a subscriber at his/her residence.

Attach a sample printed composite invoice for services billed to a University department, school, or college, or an office of the central campus administration.

Attach detailed information on the format of the composite computer-readable invoice.

Optionally, attach a description of other options for billing in addition to the above required options.

### 2.2.8.	Responsibility for Equipment and Software.

All equipment and related software required to provide the dial-up services shall be acquired, installed, and maintained by the Vendor.  It is expected that subscribers of the services will be responsible for acquiring, installing, and maintaining all personal computer equipment and related client application software to use the dial-up services.  However, Vendors may want to propose supplying software (such as scripts) to facilitate use of their services.  A Vendor's services must be compatible with a wide range of commercial and public domain access software that may be installed on a subscriber's machine.

Comply?  YES ____   NO ____

Optionally, describe any software that you are willing to supply to facilitate use of your services:

### 2.2.9.	University Responsibilities

#### 2.2.9.1.	University shall designate two University representatives to serve as vendor liaisons for administration of this contract.  One shall serve as administrative liaison and the other as technical liaison.  University's administrative liaison will serve as the point of contact for all financial and management activities.  University's technical liaison will be the single point of contact for the installation, operation and maintenance activities, including coordinating activities for informing the subscribers of the vendor's service of scheduled outages of service.  Any duties not clearly delineated between the University's administrative and technical liaisons will be initially referred to the administrative liaison for clarification of responsible party.

#### 2.2.9.2.	University shall provide secure space, electrical power and environmental control for Vendor's equipment installed on the UCSB campus.

#### 2.2.9.3.	University shall assist the Vendor or third party with marketing and promotional activities of Vendor's services, as defined in Section 1 of this RFP.

#### 2.2.9.4.	University and Vendor shall mutually develop acceptable procedures for composite billing of subscribers who receive prior approval, by the University, to have their subscription expenses invoiced to the University.

Comply?  YES ____   NO ____

### 2.2.10.	Subscriber help desk services.

Help desk services must be accessible by each subscriber via electronic mail and a toll free (800) telephone number, 24 hours per day, 365 days per year.  Telephone calls placed on hold by the help desk shall be answered within a five (5) minute maximum hold time.  Requests for subscriber support originated by electronic mail shall be answered by telephone, electronic mail or fax within twenty-four (24) hours.

The services and responsibilities of the Vendor's help desk must include the following, at a minimum:

#### 2.2.10.1.	Provides management (monitoring, configuration, problem isolation, problem correction) of Vendor's equipment and software, whether located at the UCSB campus or Vendor's facilities location.

#### 2.2.10.2.	Provides management (monitoring, configuration, problem isolation, problem correction) of the connection between the Vendor's equipment and the UCSB network.

#### 2.2.10.3.	Provides management (monitoring, configuration, problem isolation, problem correction) of the connection between the Vendor's equipment and the Internet.

#### 2.2.10.4.	Provides subscribers with any needed Internet registration, subscriber registration to Vendor's services, and subscriber authentication services.

#### 2.2.10.5.	Provides help to subscribers on how to successfully connect to the Vendor's equipment, use Internet services and connect to a host on the UCSB network.

#### 2.2.10.6.	Handles standard subscriber account inquires.

#### 2.2.10.7.	Provides a single point of contact between the Vendor and the University's technical liaison.  To ensure effective coordination of subscriber support between University technical support staff and vendor's subscriber support staff, the vendor shall provide a second, separate tier of technical support to the University technical liaison.

Comply?  YES ____   NO ____

Describe the proposed subscriber registration process:

Describe your proposed Help Desk services, in detail:

Subscriber problem resolution time must not exceed an average of forty-eight (48) hours.  Vendor must provide a detailed escalation process for problem durations that exceed the average resolution time.

Comply?  YES ____   NO ____

Describe your proposed escalation process for problem durations that exceed the average resolution time:

The Vendor shall monitor and report problem resolution statistics on a monthly basis.

Comply?  YES ____   NO ____

Vendor may, optionally, include help desk support for non-proprietary Internet access client software such as gopher, WWW, electronic mail, FTP, Telnet and tn3270.

Describe these services, if offered:

### 2.2.11.	Location of Vendor's Equipment

Vendor may propose the installation of all or portions of the equipment in the Vendor's facilities or on UCSB's campus, or both.

Will any equipment be installed on the UCSB campus:  YES ____  NO ____

If the answer to the above question is YES, describe the equipment, in detail:

If Vendor proposes the installation of any equipment on UCSB's campus, the Vendor must describe:

#### 2.2.11.1.	What maintenance services will be expected of University staff?

#### 2.2.11.2.	Will University staff be expected to respond to outages of equipment during non-business hours? If so, who will contact University staff to request services (the vendor or the subscribers of the service)?

#### 2.2.11.3.	What level of spare parts will be stored at the University site?

#### 2.2.11.4.	Who will pay for shipping and replacement of spare parts that are used by University staff to repair on-site equipment?

#### 2.2.11.5.	What type of maintenance records would University staff be expected to maintain?

#### 2.2.11.6.	What compensation will be provided to the University in exchange for the use of University's space and maintenance services?

If Vendor proposes the installation of any equipment on the University's campus, that equipment must be 19-inch rack mounted (attached to a rack with mounting screws, not set on a shelf in a rack) and will be provided by Vendor at no capital outlay costs to the University.  The Vendor is responsible for providing an uninterruptable power system (UPS) or connection to the University's -48 VDC power grid for all equipment that is located on the University's campus.  Vendor must pay all costs associated with the provision and connection of the UPS or -48 VDC power grid.  University shall provide 110 VAC electrical power for Vendor equipment installed on the UCSB campus.

Comply?  YES ____   NO ____

### 2.2.12.	System Reliability

Vendor's equipment and software must be operable and accessible by subscribers at least ninety-nine percent (99%) of the time, 24 hours per day, 365 days per year.  Vendor will compute and provide reliability reports to the University on a monthly basis. The minimum list of performance measures will be:

#### 2.2.12.1.	Number of subscriber connections, by hour of day and totaled over each day and each month.

#### 2.2.12.2.	Number of subscriber connection minutes, by hour of day and totaled over each day and each month.

#### 2.2.12.3.	Average and peak traffic (in bits/second) between the Vendor's equipment and the UCSB network, and between the Vendor's equipment and the Internet.

#### 2.2.12.4.	Round trip delay on connections between the Vendor's equipment and the UCSB network, and between the Vendor's equipment and the Internet.

#### 2.2.12.5.	Number and percentage of lost or errored packets transmitted or received over the communications circuits between the Vendor's equipment, the UCSB network and the Internet.

#### 2.2.12.6.	Time and duration of each outage of each component and communications circuit and summary of outages per month.

Comply?  YES ____   NO ____

### 2.2.13.	System Maintenance

#### 2.2.13.1.	Vendor will provide all necessary hardware and software maintenance on all equipment necessary to provide the proposed services.  The maximum response time for commencement of repair of failed hardware or software shall be two (2) hours.

#### 2.2.13.2.	Any scheduled downtime shall require a three-business-day advance notice to designated University technical liaison for short periods of downtime (less than three hours).  For scheduled periods of downtime of three hours or more, a seven-business-day advance notice to designated University technical liaison is required.

#### 2.2.13.3.	Prior to scheduling downtime, the Vendor shall coordinate with University technical liaison to ensure that the downtime is scheduled to inconvenience as few subscribers as possible.  It is expected that downtime for scheduled maintenance will generally be scheduled in non-prime time hours, and must not be scheduled during high use periods.

Comply?  YES ____   NO ____

### 2.2.14.	Internet Registration

Vendor must be responsible for all Internet registration activities, including registering IP addresses and Domain Name Servers (DNS), for all components of the system other than the UCSB network.

Comply?  YES ____   NO ____

### 2.2.15.	Security Procedures and Authentication Methods

The vendor must provide some form of authentication (preferably Kerberos-based) which will insure that only University faculty, staff, registered students and approved guests of the University will access the University's network via the direct connection between the Vendor's equipment and the University's network.  Vendor shall monitor, maintain and report on system security activities on a monthly basis.

Comply?  YES ____   NO ____

Describe the authentication process that will be used:

### 2.2.16.	Test Accounts

The Vendor shall provide the University with a minimum of three (3) test accounts on the Vendor's system, at no charge, during the term of the agreement between the Vendor and the University, for use by University's maintenance staff personnel to insure that the Vendor's services are operating correctly.

Comply?  YES ____   NO ____

### 2.2.17.	System Capacity

Vendor must maintain a modem pool capacity sufficient to accommodate an initial group of subscribers and must expand the capacity of the modem pool to meet the growth of subscribed subscribers.  Vendor must guarantee that the modem pool will provide an adequate level of modems, as measured by accessibility during peak hours of subscriber demand, such that no more than 1 out of 100 calls shall receive a busy signal during the busiest time of the day.  This means that there shall be no more than 1 busy signals for every 100 connection attempts, regardless of time of day.  Vendor will monitor usage and provide the University with reports of system capacity and utilization each month.  The primary performance measures will be:

a)	the number of modems accessible via a given calling group (i.e., pilot number) shall be determined to ensure that the entire number of modems are busy no more than 1% during the busiest hour of any day.

b)	quality of service: specify performance parameters for the services to be provided in terms of lost packets, error rate on the local line, and link speed on the connection(s) from Vendor's terminal server to the connection point to the University's network.

Comply?  YES ____   NO ____

### 2.2.18.	Tiers of Service

#### 2.2.18.1.	An "entry" subscriber level of service shall be provided.  Vendors shall indicate on the Vendor Response Form (Section 3.0) their rate for a quantity of forty (40) prime time hours of access per month, and the rate for each prime time hour beyond this figure.  Subscriber access to service during non-prime time hours shall not accumulate toward their monthly quota.

#### 2.2.18.2.	A "mid" subscriber level of service shall be provided.  Vendors shall indicate on the Vendor Response Form (Section 3.0) their rate for a quantity of sixty (60) prime time hours of access per month, and the rate for each prime time hour beyond this figure.  Subscriber access to service during non-prime time hours shall not accumulate toward their monthly quota.

#### 2.2.18.3.	A "premier" subscriber level of service shall be provided.  Vendors shall indicate on the Vendor Response Form (Section 3.0) their rate for an unlimited (unmetered) number of hours of access per month.

# 2.3	Desirable Features of Dial-up Modem Service

Items described in this section are not requirements but are of interest to the University.  This section is information only, and will not be a basis for award.  However, the features may be incorporated into the University contract, at the University's sole discretion.

### 2.3.1	Multicast Support

The Vendor may state the level of current support or implementation timelines for supporting IP multicast technology over dial-up connections.

Attach description.

### 2.3.2	Internet Services

While the University intends to provide server support for electronic mail, World Wide Web services, access to NNTP newsgroups and other Internet services, the Vendor may propose to provide these services (at a specified additional fee).

Attach description.

### 2.3.3	Client Application Software

As specified in section 2.2.8, subscribers of the Vendor's services will be responsible for acquiring, installing, and maintaining all personal computer equipment and related client application software.  Vendor's services must be compatible with a wide range of commercial and public domain access software that may be installed on a subscriber's machine.  However, University is interested in Vendor's proposals (description and cost) for a package of basic application software, including, at a minimum: telnet, FTP, WWW client, electronic mail and tn3270.

Attach description.

### 2.3.4	Client Application Software Help Desk Services

University is interested in Vendor's proposals (description and cost) for help desk services to install, configure and maintain the package of basic application software described in section 2.3.3.

Attach description.

# 2.4	Required Features of ISDN Service

### 2.4.1.	ISDN Line Rates and Channel Bonding (inverse multiplexing)

ISDN support shall include both 64 Kbps and 128 Kbps line rates on the subscriber loop.  Channel bonding, or inverse multiplexing, to achieve 128 Kbps must use Multi-Link PPP encapsulation as a minimum requirement.  Line rate requirements are for base, uncompressed connection rate.

Comply?  YES ____   NO ____

### 2.4.2.	Transport of Protocols.

Vendor's equipment must use PPP for the ISDN line protocol.  Higher level protocols that must be transported by the vendor's services include TCP, IP and UDP.  Transport of all standard application protocols, i.e., File Transfer Protocol (FTP), terminal to host connections (telnet, tn3270), electronic mail (SMTP, POP-2, POP-3), Domain Name Services (DNS), Directory Services (PH, Whois, finger), Gopher, Boot Protocol (BootP), NNTP News Services and World Wide Web (WWW) must be supported.

Comply?  YES ____   NO ____

### 2.4.3.	Local Dial-Up Access.

Subscribers of the ISDN services should not be required to call outside of a local calling zone or make a long-distance call to connect to the vendor's facilities.  The majority of potential subscribers reside within the same GTE Local Calling Area as the campus.  However, a significant portion of potential subscribers reside within the Local Access Transport Area (LATA), but outside of the GTE Local Calling Area (e.g., residents in Lompoc, Santa Maria and Carpinteria).  See Table 2 for data on where faculty, staff and students live, by Zip Code.  Vendors must specify how subscribers within the LATA but outside of the Local Calling Area will access the vendor's services without paying charges for long distance calls (e.g., by use of multiple Points of Presence, use of Foreign Exchange lines, use of a toll-free 800 number, etc.).

Comply?  YES ____   NO ____

Explanation of how requirement will be met:

### 2.4.4.	Access to the UCSB Campus Network.

Communications between the vendor's service and the UCSB network must use the IP protocol.  If the vendor's equipment is collocated at the UCSB campus, connection of the vendor's equipment to the UCSB campus network will be via an Ethernet multiport transceiver provided by UCSB.  If the vendor's equipment is located off-campus, connection of the vendor's equipment to the UCSB campus network will be via a communications circuit and an existing router installed on the UCSB campus.  The physical communications circuit between the Vendor's equipment and the UCSB equipment may be a dedicated circuit or Frame Relay service.  The Vendor is responsible for all costs to install, operate and maintain the communications circuit between the Vendor's equipment and the UCSB equipment.

Comply?  YES ____   NO ____

### 2.4.5.	Access to the Internet.

Connectivity between the vendor's service and the Internet must not traverse the UCSB campus network.  That is, the vendor must provide a dedicated connection from their service directly to the Internet for all communications with hosts other than hosts on the UCSB network.

Comply?  YES ____   NO ____

### 2.4.6.	System Topology Diagrams

Vendor shall provide the University with system topology diagrams.  The diagrams shall include information on both the ISDN equipment and software, and the Vendor's connection to the Internet.  The topological diagram of the connection to the Internet must show the path from the Vendor's local equipment to each of the national Network Access Points (NAPs), including redundant paths and redundant equipment.

Comply?  YES ____   NO ____

Attach topological diagrams of your network's connection to the Internet and paths to each of the national NAPs.

### 2.4.7.	Subscriber Billing Procedures.

Each subscriber will select the billing procedures to be used for their subscription. At a minimum, the Vendor must provide 2 billing options:

#### 2.4.7.1.	Issue a separate invoice to the subscriber at his/her residence for the Vendor's services, and

#### 2.4.7.2.	Bill a University department, school, or college, or an office of the central campus administration.  Selection of this billing option by a subscriber must require prior approval by a University employee with the appropriate funding authority.  The method for securing prior approval by a University employee shall be mutually developed and agreed upon by the Vendor and the University.  All expenses invoiced to the University will be in the form of a "composite" invoice (not individual invoices for each subscriber) and the subscriber-specific invoice detail must be provided in both printed and computer-readable (e.g., magnetic tape or disk) format.  The invoice must provide sufficient detail to allow the recipient department to assign costs to a specific individual.  The Vendor shall provide detailed information on the format of the composite computer-readable invoice.

Each subscriber must be able to individually select one of these two minimum billing options.  In addition to these two options, Vendors may propose adding charges to the subscriber's current residence telephone line invoice via arrangements with the local telephone company (GTE).  Vendors may propose other options for billing in addition to the above, as long as they satisfy the minimum billing options.

Comply?  YES ____   NO ____

Attach a sample invoice to a subscriber at his/her residence.

Attach a sample printed composite invoice for services billed to a University department, school, or college, or an office of the central campus administration.

Attach detailed information on the format of the composite computer-readable invoice.

Optionally, attach a description of other options for billing in addition to the above required options.

### 2.4.8.	Responsibility for Equipment and Software.

All equipment and related software required to provide the ISDN services shall be acquired, installed, and maintained by the service provider.  It is expected that subscribers of the services will be responsible for acquiring, installing, and maintaining all personal computer equipment and related client application software to use the ISDN services.  However, Vendors may want to propose supplying software (such as scripts) to facilitate use of their services.  A Vendor's services must be compatible with a wide range of commercial and public domain access software that may be installed on a subscriber's machine.

Comply?  YES ____   NO ____

Optionally, describe any software that you are willing to supply to facilitate use of your services:

### 2.4.9.	University Responsibilities

#### 2.4.9.1.	University shall designate two University representatives to serve as vendor liaisons for administration of this contract.  One shall serve as administrative liaison and the other as technical liaison.  University's administrative liaison will serve as the point of contact for all financial and management activities.  University's technical liaison will be the single point of contact for the installation, operation and maintenance activities, including coordinating activities for informing the subscribers of the vendor's service of scheduled outages of service.  Any duties not clearly delineated between the University's administrative and technical liaisons will be initially referred to the administrative liaison for clarification of responsible party.

#### 2.4.9.2.	University shall provide secure space, electrical power and environmental control for Vendor's equipment installed on the UCSB campus.

#### 2.4.9.3.	University shall assist the Vendor or third party with marketing and promotional activities of Vendor's services, as defined in Section 1 of this RFP.

#### 2.4.9.4.	University and Vendor shall mutually develop acceptable procedures for composite billing of subscribers who receive prior approval, by the University, to have their subscription expenses invoiced to the University.

Comply?  YES ____   NO ____

### 2.4.10.	Subscriber help desk services.

Help desk services must be accessible by each subscriber via electronic mail and a toll free (800) telephone number, 24 hours per day, 365 days per year.  Telephone calls placed on hold by the help desk shall be answered within a five (5) minute maximum hold time.  Requests for subscriber support originated by electronic mail shall be answered by telephone, electronic mail or fax within twenty-four (24) hours.

The services and responsibilities of the Vendor's help desk must include the following, at a minimum:

#### 2.4.10.1.	Provides management (monitoring, configuration, problem isolation, problem correction) of Vendor's equipment and software, whether located at the UCSB campus or Vendor's facilities location.

#### 2.4.10.2.	Provides management (monitoring, configuration, problem isolation, problem correction) of the connection between the Vendor's equipment and the UCSB network.

#### 2.4.10.3.	Provides management (monitoring, configuration, problem isolation, problem correction) of the connection between the Vendor's equipment and the Internet.

#### 2.4.10.4.	Provides subscribers with any needed Internet registration, subscriber registration to Vendor's services, and subscriber authentication services.

#### 2.4.10.5.	Provides help to subscribers on how to successfully connect to the Vendor's equipment, use Internet services and connect to a host on the UCSB network.

#### 2.4.10.6.	Handles standard subscriber account inquires.

#### 2.4.10.7.	Provides a single point of contact between the Vendor and the University's technical liaison.  To ensure effective coordination of subscriber support between University technical support staff and vendor's subscriber support staff, the vendor shall provide a second, separate tier of technical support to the University technical liaison.

Comply?  YES ____   NO ____

Describe the proposed subscriber registration process:

Describe your proposed Help Desk services, in detail:

Subscriber problem resolution time must not exceed an average of forty-eight (48) hours.  Vendor must provide a detailed escalation process for problem durations that exceed the average resolution time.

Comply?  YES ____   NO ____

Describe your proposed escalation process for problem durations that exceed the average resolution time:

The Vendor shall monitor and report problem resolution statistics on a monthly basis.

Comply?  YES ____   NO ____

Vendor may, optionally, include help desk support for non-proprietary Internet access client software such as gopher, WWW, electronic mail, FTP, Telnet and tn3270.

Describe these services, if offered:

### 2.4.11.	Location of Vendor's Equipment

Vendor may propose the installation of all or portions of the equipment in the Vendor's facilities or on UCSB's campus, or both.

Will any equipment be installed on the UCSB campus:  YES ____  NO ____

If the answer to the above question is YES, describe the equipment, in detail:

If Vendor proposes the installation of any equipment on UCSB's campus, the Vendor must describe:

#### 2.4.11.1.	What maintenance services will be expected of University staff?

#### 2.4.11.2.	Will University staff be expected to respond to outages of equipment during non-business hours? If so, who will contact University staff to request services (the vendor or the subscribers of the service)?

#### 2.4.11.3.	What level of spare parts will be stored at the University site?

#### 2.4.11.4.	Who will pay for shipping and replacement of spare parts that are used by University staff to repair on-site equipment?

#### 2.4.11.5.	What type of maintenance records would University staff be expected to maintain?

#### 2.4.11.6.	What compensation will be provided to the University in exchange for the use of University's space and maintenance services?

If Vendor proposes the installation of any equipment on the University's campus, that equipment must be 19-inch rack mounted (attached to a rack with mounting screws, not set on a shelf in a rack) and will be provided by Vendor at no capital outlay costs to the University.  The Vendor is responsible for providing an uninterruptable power system (UPS) or connection to the University's -48 VDC power grid for all equipment that is located on the University's campus.  Vendor must pay all costs associated with the provision and connection of the UPS or -48 VDC power grid.  University shall provide 110 VAC electrical power for Vendor equipment installed on the UCSB campus.

Comply?  YES ____   NO ____

### 2.4.12.	System Reliability

Vendor's equipment and software must be operable and accessible by subscribers at least ninety-nine percent (99%) of the time, 24 hours per day, 365 days per year.  Vendor will compute and provide reliability reports to the University on a monthly basis. The minimum list of performance measures will be:

#### 2.4.12.1.	Number of subscriber connections, by hour of day and totaled over each day and each month.

#### 2.4.12.2.	Number of subscriber connection minutes, by hour of day and totaled over each day and each month.

#### 2.4.12.3.	Average and peak traffic (in bits/second) between the Vendor's equipment and the UCSB network, and between the Vendor's equipment and the Internet.

#### 2.4.12.4.	Round trip delay on connections between the Vendor's equipment and the UCSB network, and between the Vendor's equipment and the Internet.

#### 2.4.12.5.	Number and percentage of lost or errored packets transmitted or received over the communications circuits between the Vendor's equipment, the UCSB network and the Internet.

#### 2.4.12.6.	Time and duration of each outage of each component and communications circuit and summary of outages per month.

Comply?  YES ____   NO ____

### 2.4.13.	System Maintenance

#### 2.4.13.1.	Vendor will provide all necessary hardware and software maintenance on all equipment necessary to provide the proposed services.  The maximum response time for commencement of repair of failed hardware or software shall be two (2) hours.

#### 2.4.13.2.	Any scheduled downtime shall require a three-business-day advance notice to designated University technical liaison for short periods of downtime (less than three hours).  For scheduled periods of downtime of three hours or more, a seven-business-day advance notice to designated University technical liaison is required.

#### 2.4.13.3.	Prior to scheduling downtime, the Vendor shall coordinate with University technical liaison to ensure that the downtime is scheduled to inconvenience as few subscribers as possible.  It is expected that downtime for scheduled maintenance will generally be scheduled in non-prime time hours, and must not be scheduled during high use periods.

Comply?  YES ____   NO ____

### 2.4.14.	Internet Registration

Vendor must be responsible for all Internet registration activities, including registering IP addresses and Domain Name Servers (DNS), for all components of the system other than the UCSB network.

Comply?  YES ____   NO ____

### 2.4.15.	Security Procedures and Authentication Methods

The vendor must provide some form of authentication (e.g., CHAP) which will insure that only University faculty, staff, registered students and approved guests of the University will access the University's network via the direct connection between the Vendor's equipment and the University's network.  Vendor shall monitor, maintain and report on system security activities on a monthly basis.

Comply?  YES ____   NO ____

Describe the authentication process that will be used:

### 2.4.16.	Test Accounts

The Vendor shall provide the University with a minimum of three (3) test accounts on the Vendor's system, at no charge, during the term of the agreement between the Vendor and the University, for use by University's maintenance staff personnel to insure that the Vendor's services are operating correctly.

Comply?  YES ____   NO ____

### 2.4.17.	System Capacity

Vendor must maintain ISDN circuit pool capacity sufficient to accommodate an initial group of subscribers and must expand the capacity of the ISDN circuit pool to meet the growth of subscribed subscribers.  Vendor must guarantee that the ISDN circuit pool will provide an adequate level of B channels, as measured by accessibility during peak hours of subscriber demand, such that no more than 1 out of 100 calls shall receive a busy signal during the busiest time of the day.  This means that there shall be no more than 1 busy signals for every 100 connection attempts, regardless of time of day.  Vendor will monitor usage and provide the University with reports of system capacity and utilization each month.  The primary performance measures will be:

a)	the number of ISDN B circuits accessible via a given calling group (i.e., pilot number) shall be determined to ensure that the entire number of B channels are busy no more than 1% during the busiest hour of any day.

b)	quality of service: specify performance parameters for the services to be provided in terms of lost packets, error rate on the local line, and link speed on the connection(s) from Vendor's terminal server to the connection point to the University's network.

Comply?  YES ____   NO ____

### 2.4.18.	Tiers of Service

#### 2.4.18.1.	An "entry" subscriber level of service shall be provided.  The communications circuit rate for this service level shall be 64 Kbps.  Vendors shall indicate on the Vendor Response Form (Section 3.0) their rate for a quantity of sixty (60) prime time hours of access per month, and the rate for each prime time hour beyond this figure.  Subscriber access to service during non-prime time hours shall not accumulate toward their monthly quota.

#### 2.4.18.2.	A "mid" subscriber level of service shall be provided.  The communications circuit rate for this service level shall be 128 Kbps.  Vendors shall indicate on the Vendor Response Form (Section 3.0) their rate for a quantity of sixty (60) prime time hours of access per month, and the rate for each prime time hour beyond this figure.  Subscriber access to service during non-prime time hours shall not accumulate toward their monthly quota.

#### 2.4.18.3.	A "premier" subscriber level of service shall be provided.  The communications circuit rate for this service level shall be 128 Kbps.  Vendors shall indicate on the Vendor Response Form (Section 3.0) their rate for an unlimited (unmetered) number of hours of access per month.

# 2.5	Desirable Features of ISDN Service

Items described in this section are not requirements but are of interest to the University.  This section is information only, and will not be a basis for award.  However, the features may be incorporated into the University contract, at the University's sole discretion.

### 2.5.1.	Residential Wiring Services.

Existing telephone wiring in the residences of faculty, staff and students may not be adequate to support ISDN services.  Vendors may provide information and prices for residential wiring services.

Attach description.

### 2.5.2.	Internet Services

While the University intends to provide server support for electronic mail, World Wide Web services, access to NNTP newsgroups and other Internet services, the Vendor may propose to provide these services (at a specified additional fee).

Attach description.

### 2.5.3.	Client Application Software

As specified in section 2.4.8, subscribers of the Vendor's services will be responsible for acquiring, installing, and maintaining all personal computer equipment and related client application software.  Vendor's services must be compatible with a wide range of commercial and public domain access software that may be installed on a subscriber's machine.  However, University is interested in Vendor's proposals (description and cost) for a package of basic application software, including, at a minimum: telnet, FTP, WWW client, electronic mail and tn3270.

Attach description.

### 2.5.4.	Client Application Software Help Desk Services

University is interested in Vendor's proposals (description and cost) for help desk services to install, configure and maintain the package of basic application software described in section 2.5.3.

Attach description.

### 2.5.5.	Vendor-provided Hardware

Vendors may choose to propose supplying their own subscriber-site hardware (i.e., ISDN adapter cards and NT-1) to facilitate use of their services.  Specifications and costs for any Vendor-specific or Vendor-recommended hardware should be provided.

Attach description.

### 2.5.6.	Local Exchange Area ISDN Usage Charges.

It is preferable that ISDN service provided to those within the GTE Local Calling Area be provided on a Centrex-based offering, with no per-call or per-minute charges.  ISDN calls associated with this service would be intra-Centrex calls.  Under this option, monthly ISDN subscription fees charged by the Vendor would include the entire subscriber loop charges.  The Vendor would be responsible for the fees that the subscriber would otherwise be required to pay to the Local Exchange Carrier; the total cost to the subscriber is fully represented in the fees the Vendor charges the subscriber for these ISDN services.

Attach description.

# 3.0	Vendor's Response Forms

Space has been provided in this document for vendors to supply price quotations, responses to specific questions, and comments.  Supporting explanatory information may be submitted and will be reviewed.  To promote compatibility among proposals and to ease the evaluation process, your response to this request must be written or typed on the RFP in the space provided or typed on separate sheets, in the same sequence as presented below.  If the responses are typed on separate sheets, each response must contain the response number and response section title (e.g., 3.1  Vendor Information).

Vendors must sign the proposal on the last page of the RFP.  Failure to follow these requirements may result in disqualification.

3.1.	 Vendor Information

Vendor Name:

Representative preparing this response:

Vendor Mailing Address:

Vendor Representative Phone Number:

Vendor Representative FAX Number:

3.2.	References

Vendors must provide a list of three (3) references to evidence their track records and performance of similar contracts.  References will preferably be for dial-in modem, ISDN and Internet services provided to colleges or universities.

3.2.1.	Reference 1:

Customer's organization name:

Address:

Telephone Number:

Contact name:

Contract start date: _______________  Contract finish date:

3.2.2.	Reference 2:

Customer's organization name:

Address:

Telephone Number:

Contact name:

Contract start date: _______________  Contract finish date:

3.2.3	Reference 3:

Customer's organization name:

Address:

Telephone Number:

Contact name:

Contract start date: _______________  Contract finish date:

Attach additional reference material, if desired.

3.3.	Financial Capabilities

Provide audited financial statements showing the condition of your firm at the end of your last two fiscal years.  This information is mandatory.  The statement must be prepared in accordance with Generally Accepted Accounting Principles (GAAP).

Dates of financial statements:

Submitted by:

Title:

Privacy notification:  The principal reason for requesting this information is to determine financial qualification as indicated in Section 1.  Final proposals and quotations are public information; however, the contents of all quotations and proposals which discloses any aspect of a vendor's quotation or proposal shall be held in confidence until notice of award.  If the vendor wishes any part of the quotation or proposal to remain confidential, a letter to that effect must be included in the Proposal.  The letter should clearly indicate which sections are to remain confidential.  Proprietary data, properly identified by the vendor, will be held in strictest confidence where there is no violation of public disclosure laws.

3.4.	Experience in university environment

Describe your firm's recent (past 2 years) experience providing similar or identical services to universities and colleges.  Use attached pages, clearly marked with the response number and title, if desired.

3.5.	Compensation.

3.5.1	Dial-up Modem Tiers of Service

3.5.1.1	Quote for "entry" subscriber level of service, as defined in section 2.2.18.1):  $_______ one-time setup fee; $_______ per month; $_______ for each prime time hour beyond forty (40) hours.

3.5.1.2	Quote for "mid" subscriber level of service, as defined in section 2.2.18.2):  $_______ one-time setup fee; $_______ per month; $_______ for each prime time hour beyond sixty (60) hours.

3.5.1.3	Quote for "premier" subscriber level of service, as defined in section 2.2.18.3):  $_______ one-time setup fee; $_______ per month.

3.5.2	ISDN Service

3.5.2.1	Quote for "entry" subscriber level of service, as defined in section 2.4.18.1):  $_______ one-time setup fee; $_______ per month; $_______ for each prime time hour beyond sixty (60) hours.

3.5.2.2	Quote for "mid" subscriber level of service, as defined in section 2.4.18.2):  $_______ one-time setup fee; $_______ per month; $_______ for each prime time hour beyond sixty (60) hours.

3.5.2.3	Quote for "premier" subscriber level of service, as defined in section 2.4.18.3):  $_______ one-time setup fee; $_______ per month.

3.5.3	Cash Discount.

The vendor agrees to provide the University with a cash discount of ________ percent for a payment within ________ days from receipt of services/materials or invoice, whichever is later, when billed to the University as a composite bill for approved subscribers.  The University cannot consider cash discounts of less than twenty (20) days.

3.5.4	Lowest offered prices.

Are the prices you have quoted herein the lowest offered to any Federal, State, Municipal or similar institution account, under similar circumstances?

Yes _____  No _____

If you checked No, please explain:

3.15.	Authorized signature

The vendor indicates full knowledge of the content of their response to this Request For Proposal and attests that they are an authorized officer of their firm, entitled to commit their firm to their response.  As a supplier of goods or services to the University of California, I/we certify that racially segregated facilities will not be maintained nor provided for employees at any establishment under my/our control and that I/we adhere to the principles set forth in Executive Orders 11248 and 11375, and undertake specifically: to maintain employment policies and practices that affirmatively promote equality of opportunity for minority group persons and women; to take affirmative steps to hire and promote women and minority group persons at all job levels and in all aspects of employment; to communicate this policy in both English and Spanish to all persons concerned within the company, with outside recruiting services, and the minority community at large; to provide the University on request a breakdown of our total labor force by ethnic group, sex, and job category; and to discuss with the University our policies and practices relating to our affirmative action program.

Firm Name:

Authorized Signature:

Printed Name:

Date:

# Table 1: Usage Statistics for Existing Pool of 128 UCSB Public Modems

# Table 2: Estimated Faculty, Staff and Student Residences, by Zip Code

| Santa Barbara County |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Faculty/Staff |  | Student |  | Total |  |
|  |  | 1994 |  | 1994 |  | 1994 |  |
| Location | Zip Code | Number | Percent | Number | Percent | Number | Percent |
| Carpinteria | 93013 | 88 | 2.14% | 58 | 0.33% | 146 | 0.67% |
| Carpinteria | 93014 | 1 | 0.02% | 1 | 0.01% | 2 | 0.01% |
| ?? | 93017 | 2 | 0.05% | 0 | 0.00% | 2 | 0.01% |
| Summerland | 93067 | 9 | 0.22% | 12 | 0.07% | 21 | 0.10% |
| Santa Barbara | 93101 | 369 | 8.96% | 724 | 4.13% | 1,093 | 5.05% |
| Santa Barbara | 93102 | 4 | 0.10% | 6 | 0.03% | 10 | 0.05% |
| Santa Barbara | 93103 | 202 | 4.91% | 201 | 1.15% | 403 | 1.86% |
| Santa Barbara | 93105 | 430 | 10.44% | 363 | 2.07% | 793 | 3.66% |
| Santa Barbara | 93106 | 94 | 2.28% | 2,718 | 15.49% | 2,812 | 12.98% |
| Santa Barbara | 93107 | 50 | 1.21% | 85 | 0.48% | 135 | 0.62% |
| Santa Barbara | 93108 | 87 | 2.11% | 65 | 0.37% | 152 | 0.70% |
| Santa Barbara | 93109 | 194 | 4.71% | 229 | 1.31% | 423 | 1.95% |
| Santa Barbara | 93110 | 267 | 6.48% | 231 | 1.32% | 498 | 2.30% |
| Santa Barbara | 93111 | 438 | 10.64% | 415 | 2.37% | 853 | 3.94% |
| Santa Barbara | 93116 | 15 | 0.36% | 9 | 0.05% | 24 | 0.11% |
| Santa Barbara | 93117 | 1,156 | 28.07% | 11,615 | 66.19% | 12,771 | 58.95% |
| Santa Barbara | 93118 | 13 | 0.32% | 11 | 0.06% | 24 | 0.11% |
| Santa Barbara | 93120 | 6 | 0.15% | 3 | 0.02% | 9 | 0.04% |
| Santa Barbara | 93121 | 12 | 0.29% | 5 | 0.03% | 17 | 0.08% |
| Santa Barbara | 93130 | 10 | 0.24% | 4 | 0.02% | 14 | 0.06% |
| Santa Barbara | 93140 | 5 | 0.12% | 1 | 0.01% | 6 | 0.03% |
| Santa Barbara | 93150 | 5 | 0.12% | 1 | 0.01% | 6 | 0.03% |
| Santa Barbara | 93160 | 14 | 0.34% | 2 | 0.01% | 16 | 0.07% |
| Santa Barbara | 93190 | 7 | 0.17% | 6 | 0.03% | 13 | 0.06% |
| Bradley | 93426 | 1 | 0.02% | 0 | 0.00% | 1 | 0.00% |
| Buellton | 93427 | 21 | 0.51% | 3 | 0.02% | 24 | 0.11% |
| Lompoc | 93436 | 133 | 3.23% | 32 | 0.18% | 165 | 0.76% |
| Lompoc | 93438 | 1 | 0.02% | 0 | 0.00% | 1 | 0.00% |
| Los Alamos | 93440 | 1 | 0.02% | 1 | 0.01% | 2 | 0.01% |
| Los Olivos | 93441 | 5 | 0.12% | 2 | 0.01% | 7 | 0.03% |
| Paso Robles | 93446 | 1 | 0.02% | 0 | 0.00% | 1 | 0.00% |
| Santa Maria | 93454 | 14 | 0.34% | 13 | 0.07% | 27 | 0.12% |
| Santa Maria | 93455 | 15 | 0.36% | 9 | 0.05% | 24 | 0.11% |
| Santa Ynez | 93460 | 22 | 0.53% | 8 | 0.05% | 30 | 0.14% |
| Solvang | 93463 | 24 | 0.58% | 6 | 0.03% | 30 | 0.14% |
| Solvang | 93464 | 1 | 0.02% | 1 | 0.01% | 2 | 0.01% |
|  | Sub-totals | 3,717 | 90.26% | 16,840 | 95.97% | 20,557 | 94.89% |
| Non-Santa Barbara County |  |  |  |  |  |  |  |  |
|  | Other Zip | 401 | 9.74% | 707 | 4.03% | 1,108 | 5.11% |
|  | Totals | 4,118 | 100.00% | 17,547 | 100.00% | 21,665 | 100.00% |